RAPID FORGE BUILDATHON

COOLSHIFT

Technical and Data Specification

SDG 7: Affordable and Clean Energy  |  SDG 13: Climate Action

PARTICIPANT DOCUMENT

48-Hour Product Development Challenge

# 1. Purpose

This document defines the required inputs, data formats, calculations, output schemas, constraints and testing volumes for the CoolShift challenge. It should be read together with the Participant Challenge Handbook and the Submission, Testing and Evaluation Guide.

Required Resolution All public evaluation data and all mandatory schedule outputs use 15-minute intervals. One day therefore contains exactly 96 records.

# 2. Public Dataset Structure

| Component | Volume |
| --- | --- |
| Public scenarios | 3 |
| Duration per scenario | 30 consecutive days |
| Interval | 15 minutes |
| Records per day | 96 |
| Records per scenario | 2,880 |
| Total public interval records | 8,640 |
| Required output window | 7 consecutive days per public scenario |
| Team-created scenario | At least 7 days / 672 interval records |
| Minimum submitted optimized outputs | 2,688 interval records across 4 scenarios |

# 3. Public Scenarios

| ID | Scenario | Purpose |
| --- | --- | --- |
| PUB-A | Household without solar | Tests affordability, peak tariff response and outage handling with limited resources. |
| PUB-B | Household with solar and battery | Tests solar utilization, battery reserve, charging/discharging and comfort trade-offs. |
| PUB-C | School or small office | Tests multi-room occupancy, daytime heat, prioritization and budget constraints. |

# 4. Required Input Domains

| Domain | Required fields |
| --- | --- |
| Run metadata | scenario_id, run_id, timezone, start timestamp, interval_minutes, algorithm_version |
| Location/weather | latitude, longitude, temperature, relative humidity, heat index/feels-like temperature, solar irradiance, sunrise/sunset where available |
| Building | building type, area, rooms/zones, insulation level, window exposure, floor level, thermal category |
| Occupancy | timestamp, occupied status or occupant count, vulnerable occupant flag, comfort priority |
| Appliances | appliance_id, type, quantity, rated power, cooling capacity, efficiency rating, minimum run time, supported setpoint range |
| Tariff/grid | tariff type, price per kWh, peak flag, grid availability, grid carbon factor |
| Solar | installed capacity, interval generation or irradiance, conversion efficiency, availability |
| Battery | capacity, initial state of charge, maximum charge/discharge rate, round-trip or directional efficiency, reserve level |
| Baseline schedule | interval appliance operation and setpoint or a deterministic rule to construct the baseline |
| User constraints | comfort range, budget, maximum demand, priority weights and selected evaluation window |

# 5. Data Types, Units and Validation

| Field | Type / unit | Validation rule |
| --- | --- | --- |
| timestamp_local | ISO 8601 datetime | Unique within scenario; sorted; exact 15-minute sequence; no silent duplicates. |
| interval_minutes | Integer minutes | Must equal 15 for mandatory evaluation. |
| temperature_c | Decimal °C | Recommended validation range −20 to 60; outliers must be flagged. |
| relative_humidity_pct | Decimal % | 0–100. |
| heat_index_c | Decimal °C | Calculated or supplied; calculation method disclosed. |
| solar_irradiance_w_m2 | Decimal W/m² | Non-negative; normally zero at night. |
| occupancy_count | Integer persons | Non-negative and consistent with building capacity. |
| grid_available | Boolean / 0 or 1 | No grid draw permitted when false/0. |
| tariff_pkr_per_kwh | Decimal PKR/kWh | Non-negative; tariff source/assumption documented. |
| grid_carbon_kgco2_per_kwh | Decimal kgCO₂e/kWh | Non-negative; factor and source/assumption documented. |
| rated_power_kw | Decimal kW | Positive for active appliances. |
| battery_soc_kwh | Decimal kWh | Must remain between 0 and capacity. |
| setpoint_c | Decimal °C | Must remain within appliance and challenge limits. |

# 6. Mandatory Input Tables

## 6.1 scenario_profiles

One row per scenario or building profile.

Required columns: scenario_id, name, timezone, building_type, area_m2, room_count, max_occupancy, insulation_level, sun_exposure, comfort_min_c, comfort_max_c, vulnerable_occupants, budget_pkr_per_day, maximum_grid_demand_kw

## 6.2 appliances

One row per appliance group.

Required columns: scenario_id, appliance_id, zone_id, appliance_type, quantity, rated_power_kw, cooling_capacity_kw, efficiency_label, min_runtime_minutes, min_setpoint_c, max_setpoint_c

## 6.3 interval_inputs

One row for every 15-minute interval.

Required columns: scenario_id, timestamp_local, temperature_c, relative_humidity_pct, heat_index_c, solar_irradiance_w_m2, occupancy_count, grid_available, tariff_type, tariff_pkr_per_kwh, grid_carbon_kgco2_per_kwh, non_cooling_load_kw

## 6.4 energy_assets

One row per scenario.

Required columns: scenario_id, solar_capacity_kw, solar_conversion_efficiency, battery_capacity_kwh, initial_soc_kwh, minimum_reserve_kwh, max_charge_kw, max_discharge_kw, charge_efficiency, discharge_efficiency

## 6.5 baseline_schedule

One row for every 15-minute interval, or generated by a fully documented baseline rule.

Required columns: scenario_id, timestamp_local, baseline_ac_units_on, baseline_ac_setpoint_c, baseline_fan_units_on, baseline_other_cooling_kw

# 7. Baseline Calculation Requirements

The baseline represents how the user currently operates cooling appliances. It must be calculated before optimization so that savings are not measured against an undefined or artificially weak comparison.

Interval appliance energy = rated power × quantity operating × interval hours.

Grid cost = grid energy × interval tariff.

Grid emissions = grid energy × interval carbon factor.

Peak demand is the maximum interval-average grid power within the evaluation period.

Solar and battery flows must be modelled explicitly if the baseline uses them.

Comfort must be estimated for the baseline using the same model used for optimized results.

Teams must disclose whether standby power, non-cooling load and conversion losses are included.

Fair Comparison Rule The baseline and optimized schedules must use the same weather, occupancy, building, appliance, tariff and comfort inputs. Only the control decisions may change.

# 8. Optimization Objective

Teams may choose any justified optimization approach. The recommended model is multi-objective: minimize cost, emissions, peak demand and discomfort while satisfying all hard constraints.

| Objective component | Examples |
| --- | --- |
| Electricity cost | Total grid cost over the planning horizon |
| Energy consumption | Cooling energy and total grid energy |
| Carbon impact | Grid energy multiplied by carbon factor |
| Peak demand | Maximum interval grid demand or energy during peak tariff periods |
| Comfort penalty | Degree-minutes outside the allowed comfort range during occupied periods |
| Battery degradation proxy | Unnecessary cycling, deep discharge or excessive charge/discharge transitions |

Teams must state whether objectives are combined through weights, priorities, lexicographic ordering, constraints or another method. User-adjustable weights are permitted, but the default configuration must be documented.

# 9. Mandatory Hard Constraints

Grid draw must equal zero when grid_available is false.

Appliance operation cannot exceed installed quantity or power capacity.

Setpoints must remain within appliance limits.

Battery state of charge must remain between zero and battery capacity.

Battery state of charge must not fall below the defined reserve unless the system explicitly declares an emergency mode.

Charge and discharge power must respect maximum rates and efficiencies.

Energy balance must hold for every interval.

Occupied-period comfort must be prioritized according to the selected user constraints.

Minimum appliance run times or switching constraints must be respected when implemented.

Missing or invalid critical inputs must not produce silent recommendations.

# 10. Comfort and Indoor Temperature Model

The challenge does not prescribe one thermal model. Teams may use a rule-based estimator, a simplified resistance-capacitance model, a machine-learning model, a calibrated simulation or another approach. The model must be consistent, documented and responsive to the following factors:

Outdoor temperature

Humidity or heat index

Building area and insulation

Sun exposure

Occupancy

Cooling capacity and appliance operation

Previous interval indoor temperature or thermal memory

Infeasible Conditions If installed cooling capacity, grid availability or stored energy cannot maintain safety, the system must report the condition instead of displaying false comfort compliance.

# 11. Required Output Schema

| Column | Requirement |
| --- | --- |
| scenario_id | Identifier matching the input scenario |
| run_id | Unique optimization run identifier |
| timestamp_local | One row per 15-minute interval |
| recommended_ac_units_on | Integer, may be zero |
| recommended_ac_setpoint_c | Numeric or blank when AC is off |
| recommended_fan_units_on | Integer, may be zero |
| grid_energy_kwh | Energy drawn from grid during interval |
| solar_energy_used_kwh | Solar energy used directly or for charging |
| battery_charge_kwh | Battery charging input during interval |
| battery_discharge_kwh | Battery output during interval |
| battery_soc_kwh | End-of-interval state of charge |
| cooling_energy_kwh | Cooling-appliance energy during interval |
| estimated_indoor_temp_c | Estimated indoor temperature |
| comfort_status | Within range, warning, unsafe or infeasible |
| interval_cost_pkr | Grid electricity cost |
| interval_emissions_kgco2e | Emissions from grid electricity |
| reason_code | Controlled code such as HEAT_RISK, SOLAR_AVAILABLE or PEAK_TARIFF |
| explanation | Human-readable reason for the decision |
| constraint_violation_count | Zero for feasible intervals; otherwise count and details |

# 12. Summary Metrics

| Metric | Calculation or expectation |
| --- | --- |
| Total cooling energy | Sum of interval cooling energy |
| Total grid energy | Sum of interval grid energy |
| Total cost | Sum of interval electricity cost |
| Total emissions | Sum of interval grid emissions |
| Peak grid demand | Maximum grid power over any interval |
| Peak-period grid energy | Grid energy consumed during marked peak intervals |
| Solar utilization rate | Solar energy used ÷ available solar energy |
| Comfort compliance | Occupied intervals within comfort range ÷ occupied intervals |
| Unsafe heat exposure | Number or duration of occupied unsafe intervals |
| Energy saving | Baseline energy − optimized energy |
| Cost saving | Baseline cost − optimized cost |
| Emission reduction | Baseline emissions − optimized emissions |

# 13. Public Output Requirement

Process the complete 30-day dataset for each public scenario without failure.

Submit seven consecutive days of optimized interval output for PUB-A, PUB-B and PUB-C.

Submit seven days of optimized interval output for one custom scenario.

Provide a summary record for every scenario and every day.

Provide one overall summary for each seven-day window.

Include zero-row gaps: missing intervals are not allowed unless clearly marked and justified.

Export machine-readable CSV or XLSX in addition to the visual dashboard.

# 14. Team-Created Custom Scenario

Each team must create one custom scenario of at least seven days (672 intervals). The custom scenario may be created through a live API, an open dataset, a reproducible synthetic generator or a documented combination of sources.

| Required item | Expectation |
| --- | --- |
| Profile | Building, users, appliances, tariff, comfort and energy assets |
| Time series | Weather, occupancy, grid availability, tariff and solar inputs at 15-minute resolution |
| Provenance | Source URLs, licences, retrieval dates or generation assumptions |
| Reproducibility | Generation script/notebook or clear repeatable procedure; random seed where applicable |
| Quality checks | Missing values, ranges, timestamp continuity and unit validation |
| Difference from public cases | At least one meaningful variation in climate, occupancy, tariff, building or energy assets |

What Is Not Required Teams are not expected to collect real sensor data for thousands of intervals during the 48-hour event. A properly documented synthetic or API-derived custom scenario is acceptable.

# 15. Missing Data and API Failure

The system must identify missing critical records before optimization.

Short gaps may be filled only through a disclosed method such as forward fill, interpolation or fallback defaults.

The original missing-data flag must be retained in logs or outputs.

API-connected systems must include a local fallback using the organizer dataset.

The live demonstration must not depend entirely on internet availability.

# 16. Performance and Reproducibility

| Test | Minimum expectation |
| --- | --- |
| Single 24-hour run | Generate 96 interval recommendations within 60 seconds on the team’s demonstration environment. |
| Seven-day run | Generate 672 interval recommendations within 5 minutes. |
| Full public import | Read and validate all 8,640 interval records. |
| Repeatability | Identical input and settings produce identical output, or a fixed seed is used and disclosed. |
| Error handling | Invalid input returns a clear validation message rather than a crash or silent value. |

# 17. Data Files Supplied

04_CoolShift_Public_Dataset_and_Templates.xlsx

Scenario profiles and appliance definitions

Thirty-day 15-minute public time series

Baseline schedules

Calculation validation cases

Output schedule and summary templates

Custom scenario template

Data dictionary and reason-code list

# Reference Sources

The challenge is aligned with the official Sustainable Development Goals. Teams may use any lawful and properly attributed data source; the following are useful starting points:

UN SDG 7: https://sdgs.un.org/goals/goal7

UN SDG 13: https://sdgs.un.org/goals/goal13

Open-Meteo weather documentation: https://open-meteo.com/en/docs

NASA POWER hourly solar and meteorological data: https://power.larc.nasa.gov/docs/services/api/temporal/hourly/